Modeling and Simulation of Trumpet Electro-mechanic Horn

2022

Vibration and Acoustic Transducers Laboratory

Pohang University of Science and Technology

Woongji Kim

wj.kim@postech.ac.kr

Table of Contents

Import horn and parallel section geometry from text file 
Acoustical components 
    Physical properties of air 
    Parallel section 
    Horn 
    Radiation 
    Compressor 
    Acoustic overall model 
Mechanical components 
    Diaphragm modeled by 3DOF linear 
    Diaphragm modeled by 1DOF nonlinear 
    Spring swtich 
Mechano-acoustical coupling 
    Diaphragm 
Electrical components 
    Coil 
    Power Supply 
    Switch 
Magnetic components 
Magneto-mechanical coupling 
Electro-mechanical coupling 
    Switch 
Mechano-acoustical coupled model

Import horn and parallel section geometry from text file

Geom_Horn = readtable("Geom_Horn.txt", opts);

Acoustical components

Physical properties of air

air.rho0    = 1.21; % Density,                  [kg/m^3]

air.c       = 343;  % Speed,                    [m/s]

air.gamma   = 1.402;% Ratio of specific heat,   [1]

air.p_ref   = 20e-6;% Reference for sound pressure level,   [Pa]

Parallel section

ac.ps.x     = Geom_Horn.Location(2);                % Length

ac.ps.s     = Geom_Horn.Area(2);                    % Area

ac.ps.n     = 20;                                   % No. of subsection

ac.ps.L     = air.rho0/ac.ps.s*(ac.ps.x/ac.ps.n);   % L in T network of subsection

ac.ps.C     = ac.ps.s/(air.rho0*air.c^2)*(ac.ps.x/ac.ps.n);

% C in T network of subsection

%% Specify the model name

ac.ps.model = 'AC_Parallel_Section';

Horn

ac.ho.x     = Geom_Horn.Location(3:end)-Geom_Horn.Location(2:end-1);

% Length of subsection

ac.ho.s     = Geom_Horn.Area(3:end);                % Area of subsection

ac.ho.n     = size(ac.ho.x);                        % No. of subsection

ac.ho.L     = air.rho0./ac.ho.s.*ac.ho.x/2;         % L in T network of subsection

ac.ho.C     = ac.ho.s/(air.rho0*air.c^2).*ac.ho.x;  % C in T network of subsection

%% Specify the model name

ac.ho.model = 'AC_Horn';

Radiation

assumed as a plane circular piston in infinite baffle

Need to implement 6th order radiation impedance

ac.rad.s    = Geom_Horn.Area(end);

ac.rad.a    = sqrt(ac.rad.s/pi);

%% Specify the model name

ac.rad.model = 'AC_Radiation';

Compressor

the value from COMSOL Multiphysics

ac.co.L     = 38361.196102405345/(2*pi*100);

ac.co.C     = 1/(2*pi*100*4.518859697974938e7);

%% Specify the model name

ac.co.model = 'AC_Compressor';

Acoustic overall model

%% Specify the model name

ac.all.model = 'AC_Overall';

Mechanical components

Diaphragm modeled by 3DOF linear

me.di.R    = 1;                     % Damping coefficient,  [N.s/m]

%% for 3DOF linear driver

me.dl.f    = [347.8491105695766 4002.8759129004957 9118.609276012086];

me.dl.phi  = [37.47048679642662 2.4855617498927827 1.8325667984395029];

%% Specify the model name

me.dl.model = 'ME_Diaphragm_3DOF_linear';

%% Specify the model name

me.dl.model = 'ME_Diaphragm_3DOF_linear_acc';

Diaphragm modeled by 1DOF nonlinear

%% for 1DOF non-linear driver

me.dn.M     = 0.028596625123380574; % Mass,                         [kg]

me.dn.Kt    = [1.028e+05    1.674e+08   2.022e+05   0   0];

% Positive spring coefficients, [N/m]

me.dn.Kc    = [6.466e+04    0   0   0   0];

% Negative spring coefficients, [N/m]

me.fd       = load('fd_nl.mat');

Spring switch

the values from the experiment result (FD curve of spring)

me.sw.K_pt  = 31e3;                 % Pretensioned-spring coefficients, [N/m]

me.sw.K     = 7e3;                  % Spring coefficients,  [N/m]

Mechano-acoustical coupling

Diaphragm

ma.di.s    = 0.0036417751495247705;

ma.di.psi  = [0.49105789017885293	4.641306076294213	0.008159872921131028];

%% Specify the model name

ma.dl.model = 'MA_Diaphragm_3DOF_linear';

Electrical components

Coil

el.coil.Ls  = 800e-6;       % Coil series inductance,       [H]

el.coil.Rs  = 670e-3;       % Coil series resistance,       [Ohm]

el.coil.Rs1 = 000e-3;       % Additional series resistance, [Ohm]

el.coil.N   = 140;          % No. of winding turns,         [1]

Power Supply

el.ps.V     = 12;           % Power supply voltage,         [V]

Switch

el.sw.R_on  = 1e-3;         % On state switch resistance,   [Ohm]

el.sw.R_off = 1e7;          % Off state switch resistance,  [Ohm]

Magnetic components

ma.arm.r    = 5.1e-3;       % Armature radius,              [m]

Magneto-mechanical coupling

mm.gap      = 1.2e-3;       % Gap distance between armature and pole,   [m]

Electro-mechanical coupling

Switch

em.sw.x_on_pt   = -70e-6;   % Transition position from spring switch free to pretensioned,  [m]

em.sw.x_off     = -360e-6;  % Transition position from spring switch pretensioned to off,   [m]

Mechano-acoustical coupled model

Sound pressure level frequency response when the armature is sinusoidally excited by 1 [N].

%% Specify the model name

ma.all.model = 'MA_Overall_3DOF_linear_driver';